Jim Foley and Josh Collins

ENGR114/Winter 2019:

3/19/19

Revision Number: 1

Hardware Hookup Guide

Hardware Setup:

Bill of Materials: (only items used/needed are listed in table)

| Component Name | Vendor and url link |
| --- | --- |
| Arduino | SparkFun RedBoard - Programmed with Arduino |
| pH Sensor | Vernier pH Sensor PH-BTA |
| Sensor Shield | SparkFun Vernier Interface Shield |
| Mini-B USB cable | SparkFun USB Mini-B Cable - 6 Foot |
| ph 4 solution | Standard Buffered pH 4 Solution - from PCC Engineering Lab |
| ph 7 solution | Standard Buffered pH 7 Solution - from PCC Engineering Lab |
| ph 10 solution | Standard Buffered pH 10 Solution - from PCC Engineering Lab |

Hardware Schematic:

Hookup Guide:

The pH sensor (Vernier part# PH-BTA) is connected to the Vernier Interface Shield (Sparkfun part# DEV-12858) using the the pH sensor's British Telecom Analog (BTA) connector. This is then plugged into either the Analog 1 or Analog 2 connector of the shield. The Vernier Interface Shield was placed on top of the Arduino (Sparkfun part# DEV-13975). The boards are aligned properly so the rows of pins on the shield and female Arduino connectors are oriented correctly. The Arduino was powered using the same red Mini-B USB cable (Sparkfun part# CAB-11301) that connects it to a laptop running Jupyter Notebook.

Images: